Good laboratory practices

- Before You Begin Good laboratory practices for PCR and RT-PCR

Precautions：To avoid contamination, perform the workflow setup under DNA-free conditions. This includes:

- Prepare and pipette all solutions with nuclease-free, DNA-free equipment and consumables.

- UV-treat or the laminar flow hood prior to pipetting.

- Use sterile single-use gloves and freshly laundered laboratory coats. Change gloves if you suspect that they are contaminated.

- Close vials immediately after pipetting. Avoid splashing or spraying samples.

- Spatial segregation of the sequential workflow steps.

| Rooms | Workflow Step |
| --- | --- |
| Sample preparation room | Extraction and purification of test samples,including preparation of recovery control sample. |
| Master mix preparation room | Master mix preparation and pipetting of PCR Negative Control to the NTC wells. |
| PCR room for setup and amplification run | Dilution and pipetting of samples and PCR Positive Control to the PCR plate. Running the PCR Instrument. |

Note：Do not use disinfectants during Sample preparation experiments so as not to affect the results.

- Avoiding false positives due to cross-contamination

To avoid false positives due to cross-contamination:

• Use a positive-displacement pipettor or aerosol‑resistant barrier pipette tips.

• Prepare and close all negative control and unknown sample tubes before pipetting the positive

control.

• Do not open tubes after amplification.

• Use different sets of pipettors when pipetting negative control, unknown, and positive control

samples.

• In the process of preparing the sample, dispense tubes in sequence from left to right the: negative controls, unknown samples and ERCs, and positive controls .